| Bus Communication with External Memory. 3rd October 2019. Prof Randall Brouwer | Daniel Ackuaku |
| --- | --- |

Figure 1 An image  of the Lab456AddressMap.xlsx file

Figure 2 A  copy of the block diagram and state diagram.

VHDL CODE for Part A

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

USE ieee.std_logic_unsigned.ALL;

ENTITY Lab4 IS

PORT (

CLOCK_50 				: IN STD_LOGIC;

KEY					: IN STD_LOGIC_VECTOR(0 DOWNTO 0);

SRAM_ADDR				: OUT STD_LOGIC_VECTOR(17 DOWNTO 0);

SRAM_DQ				: INOUT STD_LOGIC_VECTOR(15 DOWNTO 0);

SRAM_WE_N				: OUT STD_LOGIC;

SRAM_OE_N				: OUT STD_LOGIC;

SRAM_UB_N				: OUT STD_LOGIC;

SRAM_LB_N				: OUT STD_LOGIC;

SRAM_CE_N				: OUT STD_LOGIC;

ACK					: OUT STD_LOGIC;

BUS_EN					: OUT STD_LOGIC

);

END Lab4;

ARCHITECTURE Lab4_rtl OF Lab4 IS

-------------------------------------------------------------------------

-- Signal definitions

-------------------------------------------------------------------------

type State_type IS (S1,S2,S3,S4);

signal y_present, y_next : State_type;

Signal		ACK_sig 			: 	STD_LOGIC;

Signal		IRQ_sig			:  	STD_LOGIC;

Signal		Bridge_address_sig		:	STD_LOGIC_VECTOR(19 DOWNTO 0);

Signal		Bridge_bus_enable_sig	:	STD_LOGIC;

Signal		Bridge_byte_enable_sig	:	STD_LOGIC_VECTOR(1 DOWNTO 0);

Signal		Bridge_rw_sig			:	STD_LOGIC;

Signal 	Bridge_write_data_sig	:	STD_LOGIC_VECTOR(15 DOWNTO 0);

Signal 	Bridge_read_data_sig	:	STD_LOGIC_VECTOR(15 DOWNTO 0);

component nios_system is

port (

clk_clk            : in  std_logic       := 'X';           -- clk

reset_reset_n      : in  std_logic       := 'X';           -- reset_n

bridge_acknowledge : in  std_logic       := 'X';           -- acknowledge

bridge_irq         : in  std_logic       := 'X';           -- irq

bridge_address     : out std_logic_vector(19 downto 0);    -- address

bridge_bus_enable  : out std_logic;                        -- bus_enable

bridge_byte_enable : out std_logic_vector(1 downto 0);     -- byte_enable

bridge_rw          : out std_logic;                        -- rw

bridge_write_data  : out std_logic_vector(15 downto 0);     -- write_data

bridge_read_data   : in  std_logic_vector(15 downto 0) := (others => 'X')  -- read_data

);

end component nios_system;

BEGIN

-------------------------------------------------------------------------

-- Signal assigingments

-------------------------------------------------------------------------

SRAM_ADDR 	<= Bridge_address_sig(18 downto 1);

SRAM_WE_N 	<= Bridge_rw_sig;

SRAM_OE_N 	<= not Bridge_rw_sig;

SRAM_UB_N 	<= not Bridge_byte_enable_sig(1);

SRAM_LB_N 	<= not Bridge_byte_enable_sig(0);

SRAM_CE_N 	<= not (Bridge_bus_enable_sig AND Bridge_address_sig(19));

SRAM_DQ	<= Bridge_write_data_sig when Bridge_rw_sig = '0'

else (others =>'Z'); --"ZZZZZZZZZZZZZZZZ";

Bridge_read_data_sig <= SRAM_DQ;

u0 : component nios_system

port map (

clk_clk            => CLOCK_50,           	--   	clk.clk

reset_reset_n      => KEY(0),      		--  	reset.reset_n

bridge_acknowledge => ACK_sig, 			--	bridge.acknowledge

bridge_irq         => IRQ_sig,      		--     .irq

bridge_address     => Bridge_address_sig,     	--     .address

bridge_bus_enable  => Bridge_bus_enable_sig,  	--     .bus_enable

bridge_byte_enable => Bridge_byte_enable_sig, 	--     .byte_enable

bridge_rw          => Bridge_rw_sig,          	--     .rw

bridge_write_data  => Bridge_write_data_sig,  	--     .write_data

bridge_read_data   => Bridge_read_data_sig    	--     .read_data

);

-----------------------------------------------------------------------------

-- State machine process for the acknowledge

-----------------------------------------------------------------------------

process (Bridge_bus_enable_sig, y_present) is

BEGIN

CASE y_present IS

WHEN S1 =>		--State1

if (Bridge_bus_enable_sig = '1') then

ACK_sig <= '0';

y_next <= S2;

else

ACK_sig <= '0';

y_next <= S1;

end if;

WHEN S2 =>		--State2

ACK_sig <= '0';

y_next <= S3;

WHEN S3 =>		--State3

ACK_sig <= '1';

y_next <= S4;

WHEN S4 =>	--State4

ACK_sig <= '0';

y_next <= S1;

END CASE;

end process;

process (CLOCK_50, KEY(0))

BEGIN

if (KEY(0) = '0') then

y_present <= S1;

elsif (CLOCK_50' EVENT AND CLOCK_50 = '1') then

y_present <= y_next;

end if;

end process;

IRQ_sig 		<= '0';

ACK			<= ACK_sig;

BUS_EN		<= Bridge_bus_enable_sig;

END Lab4_rtl;

------------------------------------------------------------------------------------------------------------------------------------------

Figure 3 A diagram of the registers for the assembly code SRAM instructions.

Figure 4. Quartus warnings 1.

Figure 5 Quartus warnings 2.

Part C

Figure 6 Signal tap of the assembly version.

Part E

Figure 7. Register values of the c code.					Figure 8. Disassembly of the c code.