| Bus Communication with Segment Displays. 17rd October 2019. Prof. Randall Brouwer | Daniel Ackuaku |
| --- | --- |

Figure 1. A block diagram of the SRAM, OnChipRAM and SevenSegPort

Figure 2 Lab456AddressMap excel file

C CODE for Part E

#include <io.h>

#include "busbridgehal.h"

int main() {

unsigned int SRAMbase;									// SRAM base address

unsigned int OnChipRAMbase;

unsigned int mem_point;									// memory fill pointer and fill variable

unsigned int mem_fill;

unsigned int SevenSegBase;

OnChipRAMbase = 0x100200;

SevenSegBase = 0x100000;

int sub = 999;

mem_fill=200000;

mem_point=0x00;

while( mem_point <= 512){

writeBusIO(4, OnChipRAMbase, mem_point, mem_fill);

mem_fill = mem_fill-sub;

mem_point += 0x04;

}

SRAMbase = 0x1c0000;

int readPointer = 0x00;

int i = 0;

int read;

while (readPointer < 512){

read = readBusIO(4, OnChipRAMbase, readPointer);

writeBusIO(4, SRAMbase, readPointer, read);

readPointer += 0x04;

}

unsigned int HEXplus3 = 0x0;

unsigned int HEXplus6 = 0x0;

unsigned int HEXplus7 = 0x0;

unsigned int HEXsuba = 0x0;

unsigned int y = 0;

while(1){

writeBusIO(1, SevenSegBase, 0x00, HEXplus3);

writeBusIO(1, SevenSegBase, 0x01, HEXplus6);

writeBusIO(1, SevenSegBase, 0x02, HEXplus7);

writeBusIO(1, SevenSegBase, 0x03, HEXsuba);

HEXplus3 += 0x03;

HEXplus6 += 0x06;

HEXplus7 += 0x07;

HEXsuba	-=0x0a;

for (y = 0; y  < 5000000 ; y++) {

asm("nop;nop;nop;");

}

}

return 0;

}

VHDL CODE for Lab 6

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

USE work.SevenSeg_pkg.ALL;

USE ieee.std_logic_unsigned.ALL;

ENTITY Lab6 IS

PORT (

CLOCK_50 			: IN STD_LOGIC;

SW				: IN STD_LOGIC_VECTOR(0 DOWNTO 0);

KEY				: IN STD_LOGIC_VECTOR(0 DOWNTO 0);

SRAM_ADDR			: OUT STD_LOGIC_VECTOR(17 DOWNTO 0);

SRAM_DQ			: INOUT STD_LOGIC_VECTOR(15 DOWNTO 0);

SRAM_WE_N			: OUT STD_LOGIC;

SRAM_OE_N			: OUT STD_LOGIC;

SRAM_UB_N			: OUT STD_LOGIC;

SRAM_LB_N			: OUT STD_LOGIC;

SRAM_CE_N			: OUT STD_LOGIC;

HEX0				: OUT STD_LOGIC_VECTOR(6 DOWNTO 0);

HEX1				: OUT STD_LOGIC_VECTOR(6 DOWNTO 0);

HEX2				: OUT STD_LOGIC_VECTOR(6 DOWNTO 0);

HEX3				: OUT STD_LOGIC_VECTOR(6 DOWNTO 0);

HEX4				: OUT STD_LOGIC_VECTOR(6 DOWNTO 0);

HEX5				: OUT STD_LOGIC_VECTOR(6 DOWNTO 0);

HEX6				: OUT STD_LOGIC_VECTOR(6 DOWNTO 0);

HEX7				: OUT STD_LOGIC_VECTOR(6 DOWNTO 0)

);

END Lab6;

ARCHITECTURE Lab6_rtl OF Lab6 IS

-------------------------------------------------------------------------

-- Signal definitions

-------------------------------------------------------------------------

type State_type IS (S1,S2,S3,S4);

signal y_present, y_next : State_type;

Signal		ACK_sig 			:  STD_LOGIC;

Signal		IRQ_sig			:  STD_LOGIC;

Signal		Bridge_address_sig		:	STD_LOGIC_VECTOR(19 DOWNTO 0);

Signal		Bridge_bus_enable_sig	:	STD_LOGIC;

Signal		Bridge_byte_enable_sig	:	STD_LOGIC_VECTOR(1 DOWNTO 0);

Signal		Bridge_rw_sig			:	STD_LOGIC;

Signal 	Bridge_write_data_sig	:	STD_LOGIC_VECTOR(15 DOWNTO 0);

Signal 	Bridge_read_data_sig	:	STD_LOGIC_VECTOR(15 DOWNTO 0);

Signal 	q_signal			:	STD_LOGIC_VECTOR(15 DOWNTO 0);

Signal		wren_signal			:	STD_LOGIC;

Signal		wren_check			:	STD_LOGIC;

Signal		ChipEn_signal			:	STD_LOGIC;

Signal		ChipEn_check			:	STD_LOGIC;

Signal		RDData_signal			:	STD_LOGIC_VECTOR(15 DOWNTO 0);

Signal		Intermediate_signal		:	STD_LOGIC_VECTOR(15 DOWNTO 0);

Signal		HDig7_signal			:	STD_LOGIC_VECTOR(3 DOWNTO 0);

Signal		HDig6_signal			:	STD_LOGIC_VECTOR(3 DOWNTO 0);

Signal		HDig5_signal			:	STD_LOGIC_VECTOR(3 DOWNTO 0);

Signal		HDig4_signal			:	STD_LOGIC_VECTOR(3 DOWNTO 0);

Signal		HDig3_signal			:	STD_LOGIC_VECTOR(3 DOWNTO 0);

Signal		HDig2_signal			:	STD_LOGIC_VECTOR(3 DOWNTO 0);

Signal		HDig1_signal			:	STD_LOGIC_VECTOR(3 DOWNTO 0);

Signal		HDig0_signal			:	STD_LOGIC_VECTOR(3 DOWNTO 0);

component nios_system is

port (

clk_clk            : in  std_logic       := 'X';         -- clk

reset_reset_n      : in  std_logic       := 'X';         -- reset_n

bridge_acknowledge : in  std_logic       := 'X';         -- acknowledge

bridge_irq         : in  std_logic       := 'X';         -- irq

bridge_address     : out std_logic_vector(19 downto 0);  -- address

bridge_bus_enable  : out std_logic;                      -- bus_enable

bridge_byte_enable : out std_logic_vector(1 downto 0);   -- byte_enable

bridge_rw          : out std_logic;                      -- rw

bridge_write_data  : out std_logic_vector(15 downto 0);  -- write_data

bridge_read_data   : in  std_logic_vector(15 downto 0) := (others => 'X')

);

end component nios_system;

component OnChipRAM

PORT(

address	: IN STD_LOGIC_VECTOR (7 DOWNTO 0);

byteena	: IN STD_LOGIC_VECTOR (1 DOWNTO 0) :=  (OTHERS => '1');

clock		: IN STD_LOGIC  := '1';

data		: IN STD_LOGIC_VECTOR (15 DOWNTO 0);

wren		: IN STD_LOGIC ;

q		: OUT STD_LOGIC_VECTOR (15 DOWNTO 0)

);

end component;

component SevenSegPort is

Port (

Clock 	: IN std_logic;

Reset_n 	: IN std_logic;

Selct 	: IN std_logic;

CE_n 		: IN std_logic;

R_Wn 		: IN std_logic;

ADDR 		: IN std_logic_vector(2 downto 0);

ByteEn 	: IN std_logic_vector(1 downto 0);

WRData 	: IN std_logic_vector(15 downto 0);

RDData 	: OUT std_logic_vector(15 downto 0);

HDig7 	: OUT std_logic_vector(3 downto 0);

HDig6 	: OUT std_logic_vector(3 downto 0);

HDig5 	: OUT std_logic_vector(3 downto 0);

HDig4 	: OUT std_logic_vector(3 downto 0);

HDig3 	: OUT std_logic_vector(3 downto 0);

HDig2 	: OUT std_logic_vector(3 downto 0);

HDig1 	: OUT std_logic_vector(3 downto 0);

HDig0 	: OUT std_logic_vector(3 downto 0)

);

end component SevenSegPort;

BEGIN

-------------------------------------------------------------------------

-- Signal assigingments

-------------------------------------------------------------------------

SRAM_ADDR 	<= Bridge_address_sig(18 downto 1);

SRAM_WE_N 	<= Bridge_rw_sig;

SRAM_OE_N 	<= not Bridge_rw_sig;

SRAM_UB_N 	<= not Bridge_byte_enable_sig(1);

SRAM_LB_N 	<= not Bridge_byte_enable_sig(0);

SRAM_CE_N 	<= not (Bridge_bus_enable_sig AND Bridge_address_sig(19)) ;

wren_signal <= ( (NOT Bridge_rw_sig) AND wren_check);

wren_check	<= '1' when (Bridge_address_sig(19 downto 9) = "0000000001" AND Bridge_bus_enable_sig = '1') else '0';

-- tri-state buffer for write data -- 19 downto-- 0000000001

SRAM_DQ	<= Bridge_write_data_sig when Bridge_rw_sig = '0' else (others =>'Z'); --"ZZZZZZZZZZZZZZZZ";

u0 : component nios_system

port map (

clk_clk            => CLOCK_50,        --   		clk.clk

reset_reset_n      => KEY(0),          --  		reset.reset_n

bridge_acknowledge => ACK_sig, 	     -- 		bridge.acknowledge

bridge_irq         => IRQ_sig,         --       	.irq

bridge_address     => Bridge_address_sig,     	--       .address

bridge_bus_enable  => Bridge_bus_enable_sig,  	--       .bus_enable

bridge_byte_enable => Bridge_byte_enable_sig, 	--       .byte_enable

bridge_rw          => Bridge_rw_sig,          	--       .rw

bridge_write_data  => Bridge_write_data_sig,  	--       .write_data

bridge_read_data   => Bridge_read_data_sig    	--       .read_data

);

OnChipRAM_inst : OnChipRAM

PORT MAP (

address	=> Bridge_address_sig(8 downto 1),

byteena	=> Bridge_byte_enable_sig,

clock	 	=> CLOCK_50,

data	 	=> Bridge_write_data_sig,

wren	 	=> wren_signal,

q	 		=> q_signal

);

SevenSegPort_inst : component SevenSegPort

port map (

Clock 	=> CLOCK_50,

Reset_n =>	KEY(0),

Selct 	=>	SW(0),

CE_n 	=>	ChipEn_signal,

R_Wn 	=>	Bridge_rw_sig,

ADDR 	=>	Bridge_address_sig(2 downto 0),

ByteEn 	=>	Bridge_byte_enable_sig,

WRData 	=>	Bridge_write_data_sig,

RDData 	=>	RDData_signal,

HDig7 	=>	HDig7_signal,

HDig6 	=>	HDig6_signal,

HDig5 	=>	HDig5_signal,

HDig4 	=>	HDig4_signal,

HDig3 	=>	HDig3_signal,

HDig2 	=>	HDig2_signal,

HDig1 	=>	HDig1_signal,

HDig0 	=>	HDig0_signal

);

----------------------------------------------------------------------------

-- MUX for the Read data conditions

-- between the SRAM_DQ and the onChipRam q_signal

----------------------------------------------------------------------------

Intermediate_signal <= SRAM_DQ when (Bridge_bus_enable_sig NAND Bridge_address_sig(19)) = '0' else q_signal;

----------------------------------------------------------------------------

-- MUX2 for the Read data conditions

-- the result of MUX1 and the SevenSegPort RDData_signal

----------------------------------------------------------------------------

Bridge_read_data_sig <= Intermediate_signal when ChipEn_signal = '1' else RDData_signal;

-----------------------------------------------------------------------------

-- State machine process for the acknowledge

-----------------------------------------------------------------------------

process (Bridge_bus_enable_sig, y_present) is

BEGIN

CASE y_present IS

WHEN S1 =>		--State1

if (Bridge_bus_enable_sig = '1') then

ACK_sig <= '0';

y_next <= S2;

else

ACK_sig <= '0';

y_next <= S1;

end if;

WHEN S2 =>		--State2

ACK_sig <= '0';

y_next <= S3;

WHEN S3 =>		--State3

ACK_sig <= '1';

y_next <= S4;

WHEN S4 =>	--State4

ACK_sig <= '0';

y_next <= S1;

END CASE;

end process;

process (CLOCK_50, KEY(0))

BEGIN

if (KEY(0) = '0') then

y_present <= S1;

elsif (CLOCK_50' EVENT AND CLOCK_50 = '1') then

y_present <= y_next;

end if;

end process;

-----------------------------------------------------------

-- SevenSegPort CE_N_signal definition.

-----------------------------------------------------------

ChipEn_signal <= Bridge_bus_enable_sig NAND ChipEn_check;

ChipEn_check <= '1' when (Bridge_address_sig(19 downto 5) = "0000000000000000" AND Bridge_address_sig(0) = '0') else '0';

------------------------------------------------------------

IRQ_sig 		<= '0';

------------------------------------------------------------

-- Converting the register vlaues to SevenSeg

-- Hex display digits

------------------------------------------------------------

HEX7 <= convert_to_7seg(HDig7_signal);

HEX6 <= convert_to_7seg(HDig6_signal);

HEX5 <= convert_to_7seg(HDig5_signal);

HEX4 <= convert_to_7seg(HDig4_signal);

HEX3 <= convert_to_7seg(HDig3_signal);

HEX2 <= convert_to_7seg(HDig2_signal);

HEX1 <= convert_to_7seg(HDig1_signal);

HEX0 <= convert_to_7seg(HDig0_signal);

END Lab6_rtl;

Figure 3. A photo of my ID and ID num on HEX display.

VHDL Code for SevenSeg

-- Daniel Ackuaku

-- Date: 10/12/2019

Library ieee;

USE ieee.std_logic_1164.all;

-- Explanation of the entity and what it does...

-- (don't make these lines too long to avoid word wrapping)

Entity SevenSegPort is

Port (

Clock 	: IN std_logic;  -- fill in with descriptions of each port

Reset_n : IN std_logic;

Selct 	: IN std_logic;

CE_n 	: IN std_logic;

R_Wn 	: IN std_logic;

ADDR 	: IN std_logic_vector(2 downto 0);

ByteEn 	: IN std_logic_vector(1 downto 0);

WRData 	: IN std_logic_vector(15 downto 0);

RDData 	: OUT std_logic_vector(15 downto 0);

HDig7 	: OUT std_logic_vector(3 downto 0);

HDig6 	: OUT std_logic_vector(3 downto 0);

HDig5 	: OUT std_logic_vector(3 downto 0);

HDig4 	: OUT std_logic_vector(3 downto 0);

HDig3 	: OUT std_logic_vector(3 downto 0);

HDig2 	: OUT std_logic_vector(3 downto 0);

HDig1 	: OUT std_logic_vector(3 downto 0);

HDig0 	: OUT std_logic_vector(3 downto 0)

);

End Entity SevenSegPort;

Architecture Display OF SevenSegPort IS

-- This area is used to define types and any internal signals

-- Fill_In as necessary to define signals like "Result", etc

-- Defining the signals for input, output and timing control of the progamme

Signal Register1 	: std_logic_vector(31 downto 0);	-- input register

Signal Register2 	: std_logic_vector(31 downto 0); -- input reigster

Begin

-------------------------------------------------

-- Selct statement for the registers

-------------------------------------------------

Process (Selct, Register1, Register2) is

Begin

Case Selct IS

When '0' =>

HDig7 <= Register1(31 downto 28) ;

HDig6 <= Register1(27 downto 24) ;

HDig5 <= Register1(23 downto 20) ;

HDig4 <= Register1(19 downto 16) ;

HDig3 <= Register1(15 downto 12) ;

HDig2 <= Register1(11 downto 8) ;

HDig1 <= Register1(7 downto 4) ;

HDig0 <= Register1(3 downto 0) ;

When '1' =>

HDig7 <= Register2(31 downto 28) ;

HDig6 <= Register2(27 downto 24) ;

HDig5 <= Register2(23 downto 20) ;

HDig4 <= Register2(19 downto 16) ;

HDig3 <= Register2(15 downto 12) ;

HDig2 <= Register2(11 downto 8) ;

HDig1 <= Register2(7 downto 4) ;

HDig0 <= Register2(3 downto 0) ;

End Case;

End Process;

--CE-N

-----------------------------------------------------------------------------

-- process

-- reading the input into register1

-----------------------------------------------------------------------------

process (Clock, Reset_n) is

begin

if ( Reset_n = '0') then  -- if reset is pushed

Register1 <= "00000000000000000000000000000000"; -- set register1 to 0

Register2 <= "00000000000000000000000000000000"; -- set register1 to 0

elsif (Clock'EVENT AND Clock = '1' ) then

if (CE_N = '0' AND R_Wn = '0') then -- when CE_N is asserted low

-- wirte to the lower bits of Register1&2

if (ByteEn(0) = '1')then

if ( ADDR = "000" )then

Register1(7 downto 0) <= WRData(7 downto 0);

elsif ( ADDR = "010" )then

Register1(23 downto 16) <= WRData(7 downto 0);

elsif ( ADDR = "100" )then

Register2(7 downto 0) <= WRData(7 downto 0);

elsif ( ADDR = "110" )then

Register2(23 downto 16) <= WRData(7 downto 0);

end if;

end if;

-- wirte to the upper bits of Register1&2

if (ByteEn(1) = '1')then

if ( ADDR = "000" )then

Register1(15 downto 8) <= WRData(15 downto 8);

elsif ( ADDR = "010" )then

Register1(31 downto 24) <= WRData(15 downto 8);

elsif ( ADDR = "100" )then

Register2(15 downto 8) <= WRData(15 downto 8);

elsif ( ADDR = "110" )then

Register2(31 downto 24) <= WRData(15 downto 8);

end if;

end if;

end if;

end if;

end process;

-- assert read for Register 1 independently of the clock.

RDData(15 downto 0) <= Register1(15 downto 0) when ( R_Wn = '1' AND (ADDR = "000") )else

Register1(31 downto 16) when ( R_Wn = '1' AND (ADDR = "010") ) else

Register2(15 downto 0) when  ( R_Wn = '1' AND (ADDR = "100" )) else

Register2(31 downto 16) when ( R_Wn = '1' AND (ADDR = "110")) else (others => '0');

End Display;