| I/O and Interrupts. 24th October 2019. Prof. Randall Brouwer | Daniel Ackuaku |
| --- | --- |

C Code for the lab

#include "altera_up_avalon_character_lcd.h"

#include "altera_avalon_timer_regs.h"

#include "altera_up_avalon_parallel_port.h"

#include "altera_up_avalon_parallel_port_regs.h"

#include "altera_avalon_timer.h"

#include <stdio.h>

#include "sys/alt_timestamp.h"

#include "alt_types.h"

#include "system.h"

#include "altera_avalon_pio_regs.h"

#include "alt_types.h"

#include "sys/alt_irq.h"

#define KEY3_MASK 0x08  //obaitned from memeory tab in altera by pressing keys

#define KEY2_MASK 0x04

#define KEY1_MASK 0x02

/* Declare a global variable to hold the edge capture value. */

volatile int edge_capture;

static void handle_button_interrupts(void* context, alt_u32 id) {

/* cast the context pointer to an integer pointer. */

volatile int* edge_capture_ptr = (volatile int*) context;

/* Read the edge capture register on the button PIO. Store value.	*/

*edge_capture_ptr = IORD_ALTERA_AVALON_PIO_EDGE_CAP(KEY_BASE);

/* Write to the edge capture register to reset it. */

IOWR_ALTERA_AVALON_PIO_EDGE_CAP(KEY_BASE, 0x0);

/* reset interrupt capability for the Button PIO. */

IOWR_ALTERA_AVALON_PIO_IRQ_MASK(KEY_BASE, 0xf);

}

/* Initialize the button_pio. */

static void init_KEY_pio() {

/* Recast the edge_capture pointer to match the alt_irq_register() function prototype. */

void* edge_capture_ptr = (void*) &edge_capture;

/* Enable all 4 button interrupts. */

IOWR_ALTERA_AVALON_PIO_IRQ_MASK(KEY_BASE, 0xf);

/* Reset the edge capture register. */

IOWR_ALTERA_AVALON_PIO_EDGE_CAP(KEY_BASE, 0x0);

/* Register the ISR. */

alt_irq_register( KEY_IRQ,edge_capture_ptr,handle_button_interrupts );

}

static void timerInit() {

alt_avalon_timer_sc_init (TIMER_0_BASE,  TIMER_0_IRQ_INTERRUPT_CONTROLLER_ID,

TIMER_0_IRQ, TIMER_0_FREQ);

}

int main(void)

{

//Declaration of constants and variables

timerInit();

init_KEY_pio();

int flashCount;

int y;

int hexOn;

char time[8];

char copyTime[8] = "\0";

unsigned long ticks  = 0;

unsigned long resetTicks = 0;

// declaration of global vairables for the LCD, Timer and Key ports.

alt_up_parallel_port_dev * hexL;

alt_up_parallel_port_dev * hexH;

hexL = alt_up_parallel_port_open_dev ("/dev/SevenSeg30");

hexH = alt_up_parallel_port_open_dev ("/dev/SevenSeg74");

alt_up_character_lcd_dev * char_lcd_dev;

char_lcd_dev = alt_up_character_lcd_open_dev ("/dev/LCD");

while (1) {

// 1 sec = 100 ticks; 1 min = 6000 ticks; 10 mins = 60000 ticks

ticks = (alt_nticks() - resetTicks)/1;

copyTime[7] = '\0';

time[7] = '\0';

time[6] = '0'+((ticks / 10) % 10);

time[5] = '.' ;

time[4] = '0'+((ticks / 100 ) % 10);

time[3] = '0'+((ticks / 1000 ) % 6);

time[2] = ':';

time[1] = '0'+((ticks / 6000 ) % 10);

time[0] = '0'+((ticks / 60000 ) % 10);

for ( y = 0; y  < 150000 ; y++) {

asm("nop;nop;nop;");

}

alt_up_character_lcd_init (char_lcd_dev);

alt_up_character_lcd_string(char_lcd_dev, time);

alt_up_character_lcd_set_cursor_pos(char_lcd_dev, 0, 1);

alt_up_character_lcd_string(char_lcd_dev, copyTime);

alt_up_character_lcd_cursor_blink_on(char_lcd_dev);

/* Interupt Service routine for the KEY presses */

if (edge_capture & KEY1_MASK) {

//Reseting the timer

resetTicks = alt_nticks();

ticks = 0;

copyTime[0] =  0;

edge_capture = 0;

}

else if (edge_capture & KEY2_MASK) {

//Duplicating the time on the second line

strcpy(copyTime, time);

edge_capture = 0;

}

else if (edge_capture & KEY3_MASK){

//flash three times

flashCount = alt_nticks() + 300;

edge_capture = 0;

}

//Flashing the HEX displays

if (flashCount >= alt_nticks()) {

if ((flashCount - alt_nticks()) % 100 > 50){

if (hexOn == 1) {

alt_up_parallel_port_write_data(hexL, 0xffffffff);

alt_up_parallel_port_write_data(hexH, 0xffffffff);

hexOn = 0;

}

}

else {

if (hexOn == 0){

alt_up_parallel_port_write_data(hexL, 0x00000000);

alt_up_parallel_port_write_data(hexH, 0x00000000);

hexOn = 1;

}

}

}

}

return 0;

}