| Pulse Width Modulation. 30th October 2019. Prof. Randall Brouwer | Daniel Ackuaku |
| --- | --- |

#include "altera_up_avalon_character_lcd.h"

#include "altera_avalon_timer_regs.h"

#include "altera_up_avalon_parallel_port.h"

#include "altera_up_avalon_parallel_port_regs.h"

#include "altera_avalon_timer.h"

#include <stdio.h>

#include "sys/alt_timestamp.h"

#include "alt_types.h"

#include "system.h"

#include "altera_avalon_pio_regs.h"

#include "alt_types.h"

#include "sys/alt_irq.h"

#define KEY3_MASK 0x08  //obaitned from memeory tab in altera by pressing keys

#define KEY2_MASK 0x04

#define KEY1_MASK 0x02

#define AUTO 1

#define MANUAL 0

/* Declare a global variable to hold the edge capture value. */

volatile alt_32 p_Edge_Capture;

volatile alt_u64 Counter = 0;

static void HandleButtonInterrupts(void* context, alt_u32 id)

{

/* cast the context pointer to an integer pointer. */

volatile alt_32* p_Edge_Capture_Ptr = (volatile alt_32*) context;

/*

* Read the edge capture register on the button PIO.

* Store value.

*/

*p_Edge_Capture_Ptr = IORD_ALTERA_AVALON_PIO_EDGE_CAP(KEY_BASE);

/* Write to the edge capture register to reset it. */

IOWR_ALTERA_AVALON_PIO_EDGE_CAP(KEY_BASE, 0x0);

/* reset interrupt capability for the Button PIO. */

IOWR_ALTERA_AVALON_PIO_IRQ_MASK(KEY_BASE, 0xf);

}

/* Initialize the button_pio. */

static void InitKeyPio()

{

/* Recast the p_Edge_Capture pointer to match the alt_irq_register() function prototype. */

void* p_Edge_Capture_Ptr = (void*) &p_Edge_Capture;

/* Enable all 4 button interrupts. */

IOWR_ALTERA_AVALON_PIO_IRQ_MASK(KEY_BASE, 0xf);

/* Reset the edge capture register. */

IOWR_ALTERA_AVALON_PIO_EDGE_CAP(KEY_BASE, 0x0);

/* Register the ISR. */

alt_irq_register(KEY_IRQ, p_Edge_Capture_Ptr, HandleButtonInterrupts);

}

/* Initializes the timer and sets the period to 100us */

static void TimerInit()

{

IOWR(TIMER_0_BASE, ALTERA_AVALON_TIMER_PERIODL_REG, 0x1388);

IOWR(TIMER_0_BASE, ALTERA_AVALON_TIMER_PERIODH_REG, 0x0);

alt_avalon_timer_sc_init(TIMER_0_BASE, TIMER_0_IRQ_INTERRUPT_CONTROLLER_ID,

TIMER_0_IRQ, TIMER_0_FREQ);

}

/* Updates the display every 100ms */

static void RefreshDisplay(alt_8 mode, alt_up_character_lcd_dev * char_lcd_dev, alt_8 duty_cycle)

{

alt_u8 first_row[15] = "\0";

alt_u8 second_row[15] = "\0";

alt_u8 operating_mode[7] = "\0";

alt_u8 number[3] = "";

strcpy(first_row, "");

strcat(first_row, "Dutycycle = ");

number[2] = '0' + (duty_cycle / 1) % 10;

number[1] = '0' + (duty_cycle / 10) % 10;

if (duty_cycle >= 100)

{

number[0] = '1';

} else

{

number[0] = ' ';

}

strcat(first_row, number);

strcat(first_row, "%");

// Output to the LCDS

alt_up_character_lcd_init(char_lcd_dev);

alt_up_character_lcd_string(char_lcd_dev, first_row);

alt_up_character_lcd_set_cursor_pos(char_lcd_dev, 0, 1);

if (mode == AUTO)

{

strcpy(operating_mode, "Auto");

strcpy(second_row, "");

strcat(second_row, "Mode = ");

strcat(second_row, operating_mode);

}

else if (mode == MANUAL)

{

strcpy(operating_mode, "Manual");

strcpy(second_row, "");

strcat(second_row, "Mode = ");

strcat(second_row, operating_mode);

}

alt_up_character_lcd_string(char_lcd_dev, second_row);

alt_up_character_lcd_cursor_blink_on(char_lcd_dev);

}

/* Flashes the HEX displays in response to the dutyCycle */

static void PwmFunction(alt_8 duty_cycle, alt_up_parallel_port_dev * hexL)

{

//Flashing the HEX displays

if ((alt_nticks() % 100) < duty_cycle) {

alt_up_parallel_port_write_data(hexL, 0xffffffff);

}

else

{

alt_up_parallel_port_write_data(hexL, 0x00000000);

}

}

int main(void)

{

//Declaration of constants and variables

TimerInit();

InitKeyPio();

volatile alt_8 duty_cycle = 0; // The dutycyle did not update untill it was made volatile.

alt_8 mode = AUTO;

alt_8 uptick = 1;

// declaration of global vairables for the LCD, Timer and Key ports.

alt_up_parallel_port_dev * hexL;

hexL = alt_up_parallel_port_open_dev("/dev/SevenSeg30");

alt_up_character_lcd_dev * char_lcd_dev;

char_lcd_dev = alt_up_character_lcd_open_dev("/dev/LCD");

while (1)

{

PwmFunction(duty_cycle, hexL);

/* Interupt Service routine for the KEY1 press */

if (p_Edge_Capture & KEY1_MASK)

{

if (mode == AUTO)

{

mode = MANUAL;

} else if (mode == MANUAL)

{

mode = AUTO;

}

p_Edge_Capture = 0;

}

// Condition that determines counting up or down

if (duty_cycle == 100)

{

uptick = 0;

}

else if (duty_cycle == 0)

{

uptick = 1;

}

if ((Counter - alt_nticks()) <= 0)

{

if (mode == AUTO)

{

if (uptick == 1)

{

duty_cycle++;

}

else if (uptick == 0)

{

duty_cycle -= 2;

}

}

// Calls for the LCD to be updated

RefreshDisplay(mode, char_lcd_dev, duty_cycle);

Counter = alt_nticks() + 1000;

}

if (mode == MANUAL)

{

/* Interupt Service routine for the KEY2 press */

if (p_Edge_Capture & KEY2_MASK)

{

//Increasing the duty_cycle value

if (duty_cycle <= 5)

{

duty_cycle = 0;

}

else

{

duty_cycle -= 5;

p_Edge_Capture = 0;

}

}

/* Interupt Service routine for the KEY3 press */

else if (p_Edge_Capture & KEY3_MASK)

{

//Decreasing the duty_cycle value

if (duty_cycle >= 95)

{

duty_cycle = 100;

}

else

{

duty_cycle += 5;

p_Edge_Capture = 0;

}

}

}

}

return 0;

}